PRESENTER & STUDY GUIDE

Mini Project — Power Allocation in Cognitive Radio Networks

using Deep Reinforcement Learning

A detailed walkthrough of every slide, full abbreviation glossary,

deep technical background and anticipated reviewer Q&A.

Ramaiah Institute of Technology, Bangalore

Dept. of Electronics and Communication Engineering

Aditya Gangwani  ·  Ryan Gomez  ·  Shreya Revankar  ·  Sneha Tapadar

Guide: Dr. Chitra M

# 0.  How to Use This Guide

This guide is the long-form companion to the II-Review PPT. It is written for two audiences. First, the presenter — it gives a speakable script for every slide together with the key technical depth that a reviewer might probe. Second, the entire team — it works as a study document so that any team member can answer reviewer questions on any slide.

The recommended flow is:

- Skim the Glossary (Section 1) so that every acronym is fresh.

- Read Sections 2–5 once end-to-end to lock in the conceptual background.

- Use Section 6 (Slide-by-Slide Notes) the day of the review — each subsection maps to one PPT slide.

- Practise Section 7 (Anticipated Q&A) with a partner the day before.

- Use Section 8 (Quick-Reference Numbers) as a final-minute cheat sheet.

Throughout, full forms of all abbreviations appear in parentheses the first time each acronym is used.

# 1.  Glossary of Abbreviations and Key Symbols

Every abbreviation that appears in the project — in the slides, the report or the code — is listed below with its full form and a one-sentence meaning. Anything you might be asked to expand on the spot is here.

### 1.1  Wireless / Communications Acronyms

| Acronym | Full Form | Meaning in this project |
| --- | --- | --- |
| CRN | Cognitive Radio Network | Intelligent wireless network in which unlicensed Secondary Users opportunistically use licensed spectrum. |
| PU | Primary User | Licensed user with priority access — must be protected from interference. |
| SU | Secondary User | Unlicensed user who must adapt to avoid disrupting the PU. |
| PT / PR | Primary Transmitter / Primary Receiver | Endpoints of the primary direct link in our 4-node topology. |
| ST / SR | Secondary Transmitter / Secondary Receiver | Endpoints of the secondary direct link; ST chooses the transmit power. |
| CSI | Channel State Information | Knowledge of channel gains. This project uses imperfect CSI. |
| SINR | Signal-to-Interference-plus-Noise Ratio | Quality metric at a receiver — used as both observation and constraint. |
| SNR | Signal-to-Noise Ratio | Special case of SINR with no interference term. |
| BER | Bit Error Rate | Fraction of bits received in error; here computed via BPSK formula. |
| BPSK | Binary Phase-Shift Keying | Simple modulation; gives BER = 0.5·erfc(√SINR). |
| AWGN | Additive White Gaussian Noise | The noise model added at every receiver (σ² = 10⁻³ W). |
| FCC | Federal Communications Commission (USA) | Source of the 15–85 % spectrum-utilisation statistic motivating CRNs. |
| IRS | Intelligent Reflecting Surface | Tunable surface used in some literature-survey papers. |
| NOMA | Non-Orthogonal Multiple Access | Multi-user technique referenced in one of the lit-survey papers. |
| OFDM | Orthogonal Frequency-Division Multiplexing | Multi-carrier technique listed under future work. |
| MIMO | Multiple-Input Multiple-Output | Multi-antenna setup; future-work extension. |
| D2D | Device-to-Device communication | Subject of one of the lit-survey papers. |
| WPCN | Wireless Powered Communication Network | Subject of one of the lit-survey papers. |
| DSA | Dynamic Spectrum Access | Umbrella term for CRN-style spectrum sharing (e.g., IEEE 802.22). |
| QoS | Quality of Service | Service-level metric the SU must respect for the PU. |
| dB | Decibel | Logarithmic unit; 0 dB ≈ ratio of 1, 10 dB ≈ ratio of 10, etc. |

### 1.2  Machine Learning / RL Acronyms

| Acronym | Full Form | Meaning in this project |
| --- | --- | --- |
| RL | Reinforcement Learning | ML paradigm where an agent learns a policy by trial-and-error. |
| DRL | Deep Reinforcement Learning | RL with neural networks as function approximators. |
| MDP | Markov Decision Process | Formal model of RL: (S, A, P, R, γ). |
| CMDP | Constrained Markov Decision Process | MDP plus inequality constraints; theoretical foundation of the Proposed Algorithm. |
| DDPG | Deep Deterministic Policy Gradient | Baseline 1 (Lillicrap et al., 2015) — single critic, OU noise. |
| TD3 | Twin Delayed Deep Deterministic Policy Gradient | Baseline 2 (Fujimoto et al., 2018) — twin critics, delayed updates, target smoothing. |
| DQN | Deep Q-Network | Discrete-action value-based method; competitor that the project moves away from. |
| SAC | Soft Actor-Critic | Entropy-regularised continuous-control method; listed under future work. |
| PPO | Proximal Policy Optimization | On-policy actor-critic baseline; mentioned in RL families. |
| TRPO | Trust Region Policy Optimization | Predecessor of PPO; basis for Constrained Policy Optimization. |
| MARL | Multi-Agent Reinforcement Learning | Future-work extension for multiple SU pairs. |
| GRU | Gated Recurrent Unit | Recurrent neural network cell used in the belief encoder of the Proposed Algorithm. |
| LSTM | Long Short-Term Memory | Alternative recurrent cell; rejected here because it is heavier than GRU at L = 8. |
| OU | Ornstein–Uhlenbeck (noise process) | Temporally-correlated exploration noise used by DDPG. |
| TD | Temporal Difference | Update rule that bootstraps value estimates one step ahead. |
| UTD | Update-to-Data ratio | Number of gradient updates per environment step (we use 2). |
| TF32 | TensorFloat-32 | Reduced-precision GPU math mode that ~2× speeds training on RTX 30/40 series. |
| Adam | Adaptive Moment Estimation (optimiser) | The optimiser used for actor and critic networks. |
| MSE | Mean Squared Error | Critic loss form. |
| i.i.d. | independent and identically distributed | Property assumed for per-step channel draws in block fading. |

### 1.3  Software / Engineering Acronyms

| Acronym | Full Form | Meaning in this project |
| --- | --- | --- |
| API | Application Programming Interface | We expose a Gym-like API on the CRN environment. |
| WebSocket | (Full-duplex web protocol) | Pushes live training metrics from the FastAPI backend to the browser. |
| CUDA | Compute Unified Device Architecture (NVIDIA) | GPU compute platform we use via PyTorch. |
| GPU | Graphics Processing Unit | Parallel compute hardware; we use RTX 4060 / T4. |
| CPU | Central Processing Unit | Fallback compute path. |
| PDF | Portable Document Format | Reports auto-generated by Matplotlib PDFPages. |
| YAML / JSON | YAML Ain't Markup Language / JavaScript Object Notation | Common config file formats (we use plain .py). |

### 1.4  Key Symbols

| Symbol | Meaning |
| --- | --- |
| P_p | Primary transmit power (fixed, 1.0 W) |
| P_s | Secondary transmit power (the action, 0 to P_max) |
| P_max | Maximum allowed SU transmit power (3.0 W) |
| ¦h_pp¦² | Channel power gain on the PT→PR direct link |
| ¦h_ss¦² | Channel power gain on the ST→SR direct link |
| ¦h_sp¦² | Channel power gain on the ST→PR interference link |
| ¦h_ps¦² | Channel power gain on the PT→SR interference link |
| σ² | AWGN noise power at every receiver (10⁻³ W) |
| m | Nakagami fading severity parameter (1 or 3 in this work) |
| Ω | Nakagami mean link power (1.0) |
| γ_th | PU SINR protection threshold (1.0 linear, ≈ 0 dB) |
| α, β, γ_e | Scalar reward weights (throughput / PU protection / energy) used by DDPG and TD3 |
| λ₁, λ₂, λ₃ | Adaptive Lagrangian weights learned online by the Proposed Algorithm |
| τ | Polyak averaging rate for target networks (0.005) |
| γ | RL discount factor (0.99) — distinct from γ_e and γ_th |
| b_t | Belief vector output by the GRU encoder, dim 16 |
| s_t, a_t, r_t | State, action and reward at time t |
| π_φ(s) | Actor policy parameterised by φ |
| Q_θ(s, a) | Critic Q-value parameterised by θ |

# 2.  Cognitive Radio Networks — Conceptual Deep Dive

## 2.1  Why Cognitive Radios Exist

Spectrum is finite. Regulatory bodies (such as the FCC) allocate frequency bands to licensed services — TV, cellular, military, satellite. Measurement studies (FCC 2002 and follow-ups) showed that even with these licensed allocations, actual usage at any given time and place sits between 15 % and 85 %. The spectrum is not scarce; it is poorly utilised. Cognitive Radios fix this by letting unlicensed Secondary Users opportunistically transmit on licensed bands, provided they do not harm the licensed Primary User.

## 2.2  The Three Operational Paradigms

### Underlay

The SU transmits simultaneously with the PU, but keeps its interference at the PR below a pre-defined threshold. This requires accurate channel-state information and continuous power control — the SU must constantly choose its transmit power so that the PU's SINR stays above the protection threshold. Underlay gives continuous SU access and is the paradigm used in this project.

### Overlay

The SU has prior knowledge of the PU's codebook or message. It splits its transmit power between its own data and relaying / pre-coding for the PU (techniques such as dirty-paper coding). The PU can actually benefit from the SU's help. This is mostly a theoretical upper bound because the cooperation assumption is strong.

### Interweave

The SU transmits only when it has sensed the PU absent — exploiting spectrum holes. It relies entirely on accurate spectrum sensing (energy detection, matched filtering or cyclostationary detection). The interference to the PU is zero if sensing is perfect, but SU throughput drops sharply when the PU is active. Standards such as IEEE 802.22 (TV white spaces) operate in this mode.

## 2.3  Why Power Control is the Hard Sub-problem of Underlay

In underlay, the SU's challenge collapses to: choose P_s every channel use so that I maximise my own throughput log₂(1 + SINR_s) while keeping SINR_p above γ_th. Three things make this hard:

- Random channels — every link gain |h|² is drawn fresh from a Nakagami distribution at every step.

- Coupled SINRs — increasing P_s helps SINR_s linearly but hurts SINR_p linearly (through |h_sp|²); the trade-off is state-dependent.

- Multi-objective — we also want to save energy and avoid violating QoS constraints; a single scalar penalty mis-prioritises.

This is why deep RL fits: it can learn a policy P_s = π(s) over a continuous high-dimensional state space without needing a closed-form solution to the stochastic non-convex problem.

## 2.4  The Nakagami-m Fading Model in Detail

Wireless channels fluctuate due to multipath, scattering and shadowing. The Nakagami-m model captures these effects through a single parameter m that controls the severity of fading. The channel power gain follows a Gamma distribution:

|h|² ~ Gamma(shape = m, scale = Ω / m),    E[|h|²] = Ω

Special cases — m = 1 reduces to a Rayleigh channel (severe small-scale fading, no line-of-sight); m = 3 represents moderate fading typical of partial line-of-sight; m → ∞ approaches an AWGN-only channel (no fading). In our project Ω = 1.0 (unit mean power) and σ² = 10⁻³ W (noise floor).

Block fading means the gain stays constant for one channel use (one RL time step) and is re-drawn independently next step. This is a worst-case temporal-correlation assumption — it makes the channels Markov so RL theory applies cleanly.

## 2.5  Imperfect CSI

Channel State Information is what the agent knows about the channels. Perfect CSI means the agent sees |h|² exactly. Imperfect CSI — the realistic case used throughout our experiments — means the agent sees a noisy estimate. Practical receivers estimate channels from pilot symbols, so estimation noise is inevitable. Designing an agent that performs well under imperfect CSI is more valuable than one that needs an oracle channel observation.

# 3.  Reinforcement Learning — Foundations

## 3.1  The Big Picture

Reinforcement Learning is a paradigm for sequential decision-making. An agent interacts with an environment at discrete time steps. At each step it observes a state s_t, chooses an action a_t, receives a scalar reward r_t and moves to a new state s_{t+1}. The agent's job is to learn a policy π that maximises the expected cumulative discounted reward.

J(π) = E_τ~π [ Σ_t γ^t · r_t ]

γ ∈ [0, 1] is the discount factor that values short-term reward over long-term. We use γ = 0.99 — meaning the agent considers rewards roughly 100 steps into the future.

## 3.2  Markov Decision Process (MDP)

Formally, an RL problem is a tuple (S, A, P, R, γ):

- S — state space (we use 7-dimensional continuous)

- A — action space (we use 1-dimensional continuous, P_s ∈ [0, P_max])

- P(s′ | s, a) — transition probability (induced by the channel model)

- R(s, a) — reward function (combines throughput / interference / energy)

- γ — discount factor

Markov property — future depends only on the current state, not the full history. In our environment this holds because channel gains are drawn i.i.d. per step and P_s(t−1) is included in the state.

## 3.3  Value Functions

Two fundamental quantities are learned by RL:

V^π(s) = E_π [ Σ_t γ^t r_t | s_0 = s ]        (state value)

Q^π(s, a) = E_π [ Σ_t γ^t r_t | s_0 = s, a_0 = a ]  (state-action value)

Algorithms that learn Q^π are called value-based (Q-learning, DQN). Algorithms that learn π directly are policy-based (REINFORCE, Policy Gradient). Actor-critic methods (DDPG, TD3, SAC, PPO) learn both — an actor produces actions and a critic evaluates them.

## 3.4  The Bellman Equation

The recursive identity that all value-based methods exploit:

Q^π(s, a) = E [ r + γ · Q^π(s′, π(s′)) ]

Critics in DDPG, TD3 and the Proposed Algorithm are trained to satisfy this equation by minimising the Mean Squared Error between the network's prediction Q(s, a) and the bootstrapped target r + γ · Q_target(s′, a′).

## 3.5  Exploration vs Exploitation

If the agent always acts greedily it might never discover better actions. Exploration is the deliberate injection of noise into the action so the agent can find improvements. DDPG uses Ornstein-Uhlenbeck noise (temporally correlated). TD3 uses symmetric Gaussian. The Proposed Algorithm uses biased Gaussian — see Section 5.4.

## 3.6  On-policy vs Off-policy

On-policy methods (REINFORCE, PPO) learn from data generated by the current policy. Off-policy methods (DDPG, TD3, the Proposed Algorithm) reuse data from any past policy via a replay buffer. Off-policy is far more sample-efficient and is essential for fair comparison of three different algorithms on the same channel realisations.

# 4.  DDPG — The Baseline (Phase 1 Carry-Over)

## 4.1  Where DDPG Comes From

DDPG was proposed by Lillicrap et al. (ICLR 2016). It combines the actor-critic structure with two ideas borrowed from DQN: a replay buffer (for off-policy sample efficiency) and target networks (for stability). It was the first deep RL method to convincingly solve continuous-action MuJoCo benchmarks.

## 4.2  Architecture

Two neural networks:

- Actor π_φ(s) — outputs a deterministic action. Sigmoid output × P_max to bound P_s ∈ [0, P_max]. Two hidden layers, width 512, ReLU.

- Critic Q_θ(s, a) — outputs a scalar Q-value. Same depth/width.

Two target networks π_φ′ and Q_θ′ are slow-moving copies updated by Polyak averaging: φ′ ← τ·φ + (1−τ)·φ′ with τ = 0.005. Target networks stabilise the bootstrapped Bellman target so the critic does not chase a moving goal.

## 4.3  Update Equations

y = r + γ · Q_θ′(s′, π_φ′(s′))                          [TD target]

L_critic = E [ (Q_θ(s, a) − y)² ]                          [MSE]

∇_φ J = E [ ∇_a Q_θ(s, a) |_{a = π_φ(s)} · ∇_φ π_φ(s) ]   [deterministic policy gradient]

## 4.4  Exploration — Ornstein-Uhlenbeck Noise

OU noise is a temporally-correlated stochastic process:

dx_t = θ(μ − x_t) dt + σ dW_t

Unlike Gaussian noise, OU has momentum — successive noise samples are correlated, encouraging exploration in one direction for several steps. This was the standard choice in DDPG papers, but in our CRN setting OU occasionally drives P_s to extreme values that violate the PU constraint.

## 4.5  Known Weaknesses (Why TD3 Was Needed)

- Q-value overestimation — a single critic with max-operator targets systematically over-estimates Q, biasing the actor toward over-confident actions.

- Sensitivity to hyper-parameters — small changes in α, β, γ_e or learning rates can collapse the policy.

- Instability under Nakagami — reward variance under heavy fading amplifies critic noise.

# 5.  TD3 — The Stronger Baseline

## 5.1  Origin

TD3 was proposed by Fujimoto, Hoof and Meger (ICML 2018). It is the most empirically-validated off-policy continuous-control algorithm of its generation and is the direct starting point for the Proposed Algorithm.

## 5.2  Three Independent Improvements over DDPG

### Twin Critics — Clipped Double Q-Learning

Two critic networks Q_θ1 and Q_θ2 are trained in parallel. The target uses the smaller of the two Q-values:

y = r + γ · min_{i = 1, 2} Q_{θ_i′}(s′, a′)

Taking the min eliminates the positive overestimation bias of DDPG — a tighter, safer Q-estimate guides the actor.

### Delayed Policy Updates

The actor is updated once for every d = 2 critic updates. Reasoning: a bad critic produces a bad actor gradient. Letting the critic train for 2 steps before updating the actor means the actor sees more reliable gradients.

### Target Policy Smoothing

Clipped Gaussian noise is added to the target action before the critic evaluates it:

a′ = clip(π_φ′(s′) + ε, 0, P_max),   ε ~ clip(N(0, σ²), −c, +c)

Smoothing acts as a regulariser — it discourages the critic from exploiting sharp spikes in Q (which would propagate into noisy actor gradients). We use σ = 0.2·P_max and c = 0.5·P_max.

## 5.3  What TD3 Still Lacks (Why the Proposed Algorithm)

- Scalar reward — α, β, γ_e are still fixed hyper-parameters; the agent cannot rebalance them online when channel conditions change.

- No constraint guarantee — the hinge penalty is soft; nothing prevents occasional PU violations if they are throughput-profitable.

- Memoryless state — the agent ignores temporal patterns (recent fades, action consequences over the last few steps).

- Symmetric exploration — roughly half of exploration goes to higher (less safe) powers, wasting safe-side exploration.

# 6.  The Proposed Algorithm — In Depth

The Proposed Algorithm is a constrained, adaptive, multi-objective extension of TD3 designed specifically for the CRN power-allocation problem. It introduces four orthogonal innovations on top of TD3.

## 6.1  Innovation 1 — GRU Belief Encoder

A Gated Recurrent Unit (GRU) is a recurrent neural network cell with two gates (update and reset) that decide what information to carry from the past hidden state into the next. The structure is:

z_i = σ(W_z [h_{i−1}, x_i])         [update gate]

r_i = σ(W_r [h_{i−1}, x_i])         [reset gate]

ñ_i = tanh(W_n [r_i ⊙ h_{i−1}, x_i])

h_i = (1 − z_i) ⊙ h_{i−1} + z_i ⊙ ñ_i

In the Proposed Algorithm a 2-layer GRU with hidden size 64 processes the last SEQ_LEN = 8 observations and outputs a 16-dimensional belief vector b_t = W_proj · h_L + c_proj. The actor and all six critics consume the concatenation [s_t, b_t]. The belief vector lets the agent represent short-term temporal patterns the 7-D static state cannot capture.

Why GRU and not LSTM or Transformer? GRU has fewer parameters than LSTM and matches it empirically on small sequences. Transformers need longer sequences (≈ 32+) to outperform recurrent cells; at SEQ_LEN = 8, GRU is strictly the better choice. Why SEQ_LEN = 8? Long enough to capture short-term action consequences, short enough to fit easily in GPU memory; empirically tuned across 4 / 8 / 16.

## 6.2  Innovation 2 — Decomposed Multi-Objective Critics

Instead of a single scalar reward, the Proposed Algorithm decomposes the reward into a 3-vector:

- r^tput = α · log₂(1 + SINR_s)  ≥ 0   (maximise)

- r^intf = − β · max(0, γ_th − SINR_p)  ≤ 0   (keep near zero)

- r^energy = − γ_e · (P_s / P_max)  ≤ 0   (minimise magnitude)

A separate twin-critic pair is trained per objective — six critics in total. Each critic learns a clean per-objective Q-value, free from mixing with the other terms.

y_k = r_t^k + γ · min_{j = 1, 2} Q_{θ_j, k}′(s′, b′, ã′),   k ∈ {tput, intf, energy}

Why decompose? Throughput contributions in our setting are on the order of +2, while interference penalties can reach −15. A single critic learning the sum is dominated by the interference term, making it hard to distinguish a low reward caused by losing throughput from one caused by hitting the constraint. Decomposition makes the signals cleanly separable, enabling the Lagrangian (Innovation 3) to trade them off explicitly.

## 6.3  Innovation 3 — Adaptive Lagrangian Weights

The actor maximises a weighted combination of the three Q-values:

L_actor = − E [ λ₁ · Q^tput(s, b, a) + λ₂ · Q^intf(s, b, a) + λ₃ · Q^energy(s, b, a) ]

Crucially, the weights λ_k are not fixed hyper-parameters — they are learned online via dual gradient ascent and clamped to [λ_min, λ_max] = [0.1, 20.0]:

λ_k ← clip( λ_k + η_λ · ∇_{λ_k} L , λ_min , λ_max )

Positivity is enforced via softplus parameterisation: λ_k = log(1 + exp(log_λ_k)). The intuition — if PU violations become frequent, the violation signal grows, ∇_{λ_2} L becomes positive, λ_2 increases, and the actor's loss penalises interference more. If throughput collapses, λ_1 climbs to restore SU rate. The agent discovers the right balance autonomously, removing the need to hand-tune α, β, γ_e for every scenario.

Theoretical basis — this is the primal-dual formulation of the constrained problem (Boyd & Vandenberghe, 2004, Convex Optimization, Ch. 5). The same framework underlies Constrained Policy Optimization (Achiam, 2017) and Safety-Gym benchmarks (OpenAI, 2019).

## 6.4  Innovation 4 — Directional Exploration Noise

Standard TD3 explores symmetrically: ε ~ N(0, σ²). Half the explored actions push P_s up (more interference) and half down (less interference). Early in training many violations occur as the agent samples high-power actions blindly.

The Proposed Algorithm replaces the zero-mean Gaussian with a biased Gaussian whose mean μ pushes actions toward safer (lower-power) regions when violations are frequent:

μ_t = μ_bias · decay(t) · (1 + v_t)

decay(t) = max(0, 1 − t / T_decay)

ε ~ N(μ_t, σ²)

μ_bias = −0.05 is a small negative number (pushes toward lower power); v_t is the rolling violation rate over the last 100 steps (multiplies the bias when violations rise); decay(t) shrinks the bias linearly over T_decay ≈ 200 000 steps so the final policy is undistorted. The mechanism is conceptually a 'safe exploration' technique borrowed from the robotics safe-RL literature (García & Fernández, 2015).

## 6.5  Why the Proposed Algorithm Wins

- Lower outage — adaptive λ_2 auto-amplifies when violations occur; the agent self-corrects without hand-tuning.

- Higher SU throughput at the same outage — decomposed critics give the actor cleaner gradients; it can extract more throughput in benign states.

- Better adaptation — the GRU lets the policy exploit short-term channel patterns.

- Safer exploration — fewer catastrophic early-training violations means faster convergence to the constraint-satisfying region.

## 6.6  Acknowledged Trade-offs

- ≈ 2.5× more parameters (6 critics + GRU).

- ≈ 8× longer training time per episode.

- Slower initial convergence — the Lagrangians need to stabilise before the policy can converge.

- Inference cost is unchanged — only training scales up. Once trained, deployment is identical to TD3.

# 7.  Slide-by-Slide Presenter Notes

Each subsection below corresponds to one PPT slide. For each slide we list (i) the title, (ii) what to say in 30–60 seconds, (iii) the technical depth a reviewer might dig into, and (iv) the one or two killer points to land.

## Slide 1 — Title Page

Speak

Good morning. We are presenting our Mini Project on Power Allocation in Cognitive Radio Networks using Deep Reinforcement Learning, under the guidance of Dr. Chitra M. The project is being carried out by Aditya, Ryan, Shreya and Sneha from the 6th-semester ECE batch.

Why this title

'Power Allocation' is the action; 'Cognitive Radio Networks' is the system; 'Deep Reinforcement Learning' is the method. The subtitle 'A Learning-Based Approach to Adaptive Decision-Making' generalises the contribution beyond just this one problem.

## Slide 2 — Table of Contents

Speak

Walk the reviewers through the seven sections in one sentence each so they know what to expect: Introduction → Problem Statement → Proposed Algorithm → Implementation → Results → Conclusion → References.

## Slide 3 — Introduction: CRN + Three Paradigms

Speak

A Cognitive Radio Network lets unlicensed Secondary Users opportunistically use licensed spectrum without harming Primary Users. There are three paradigms — Underlay, Overlay, Interweave. We focus on Underlay because it is the most general and continuously-active, and because power control is its central sub-problem.

Killer point

Cite FCC 2002 — licensed spectrum is under-utilised 15–85 % of the time. CRNs exist to recover that unused capacity.

Reviewer might ask

Why not interweave? Answer: interweave needs perfect sensing and gives zero rate when the PU is active; underlay gives continuous SU access with the trade-off that the SU must precisely control its power.

## Slide 4 — Introduction: Reinforcement Learning

Speak

RL is the branch of ML where an agent learns by interacting with an environment to maximise a reward. The problem is formalised as a Markov Decision Process. We use Deep RL because the state and action spaces in our problem are continuous and high-dimensional, ruling out tabular methods.

Killer point

Three RL families — value-based (DQN), policy-based (REINFORCE), and actor-critic (DDPG, TD3). Our project is actor-critic, continuous-action, off-policy.

## Slide 5 — Problem Statement

Speak

CRNs need dynamic power allocation. Existing DQN methods use discrete actions, which are too coarse for fine-grained power control under fading. We need a continuous-action, constraint-aware learning framework.

Killer point

Four challenges — continuous control, heterogeneous interference, QoS constraints, non-stationarity — drive the entire algorithm design that follows.

## Slide 6 — Objectives

Speak

Three objectives — (1) build a continuous-action learning framework, (2) incorporate constraint awareness, (3) evaluate spectrum efficiency, interference management and convergence. The footer maps each objective to a concrete technical mechanism in our Proposed Algorithm.

## Slide 7 — Recap: From Self-Driving Cars to CRN

Speak

TD3 was originally designed for autonomous-driving and MuJoCo benchmarks. The car has a continuous steering / throttle action, observations like lidar and velocity, and a reward of distance-minus-collision. Our CRN problem is structurally identical — continuous P_s action, observations are channel gains and SINRs, reward is throughput minus PU violation penalty. The same TD3 machinery transfers with minimal redesign.

Killer point

If the reviewer asks 'why TD3?', this analogy nails it — it is the best-validated continuous-control algorithm in the literature and the constraints map directly.

## Slide 8 — System Model

Speak

Four nodes — PT, PR, ST, SR — with four channel gains: |h_pp|² and |h_ss|² on the direct links, |h_sp|² and |h_ps|² on the interference links. Channels follow Nakagami-m block fading; m = 1 is Rayleigh, m = 3 is moderate. The SU chooses P_s, computes SINR for both receivers, and the reward function balances throughput, PU protection and energy.

Reviewer might ask

Why fixed PU power? Answer: standard assumption that the PU is unaware of the SU and transmits at its nominal level; the SU adapts.

## Slide 9 — MDP Formulation

Speak

State is 7-dimensional: four channel gains, two SINRs, last action. Action is continuous P_s ∈ [0, P_max = 3 W]. The agent observes only imperfect CSI — the realistic deployment scenario.

Killer point

Including P_s(t−1) in the state enforces the Markov property when channels are i.i.d., and gives the agent action-continuity / momentum information.

## Slide 10 — Algorithm Evolution

Speak

We progressed through three algorithms. DDPG was the Phase-1 baseline — single critic, OU noise, often unstable on Nakagami-m=3. TD3 fixed DDPG with twin critics, delayed updates and target smoothing — faster and smoother but still scalar reward. Our Proposed Algorithm extends TD3 with four targeted innovations.

Killer point

Each algorithm directly addresses a failure mode of the previous one — this is a principled engineering progression, not a wholesale redesign.

## Slide 11 — DDPG Recap

Speak

DDPG is an actor-critic for continuous actions. Walk through the four equations on the slide (target, critic loss, actor gradient, Polyak update). Mention OU noise. Conclude with the inference — DDPG worked but its instability under Nakagami-m fading motivated the move to TD3.

## Slide 12 — TD3 Recap

Speak

TD3 fixes three things: twin critics + min target eliminates overestimation; delayed actor updates reduce gradient noise; target policy smoothing acts as a critic regulariser. The four limitations listed on the right (scalar reward, no constraint guarantee, memoryless state, symmetric noise) are exactly what the Proposed Algorithm addresses.

## Slide 13 — Proposed Algorithm Overview

Speak

Four orthogonal innovations on top of TD3. (1) GRU belief encoder for temporal context. (2) Decomposed multi-objective critics — 6 critics learning per-objective Q-values. (3) Adaptive Lagrangian weights learned by dual gradient ascent. (4) Directional exploration noise biased toward safer power levels.

## Slide 14 — Innovations 1 & 2

Speak

The GRU compresses 56 raw observations (8 × 7) into a 16-D belief vector that captures short-term channel trends. The decomposed critics turn the scalar reward into a 3-vector and learn 6 critics in total. Each per-objective Q-value is clean and separable.

Reviewer might ask

Why GRU and not LSTM? Answer: fewer parameters, matches LSTM at SEQ_LEN = 8; LSTM would be over-parameterised for a short history.

## Slide 15 — Innovations 3 & 4

Speak

Adaptive Lagrangian — λ₁, λ₂, λ₃ are learned by dual gradient ascent and clamped to [0.1, 20]. The actor maximises λ₁Q^tput + λ₂Q^intf + λ₃Q^energy. When violations rise, λ₂ grows automatically and the actor penalises interference more. Directional noise — biased Gaussian whose mean pushes toward safer actions; the bias decays as the agent matures.

Killer point

Adaptive Lagrangian is the standard primal-dual formulation from Boyd & Vandenberghe. We are not inventing maths; we are applying classical convex-optimisation duality inside the actor loss.

## Slide 16 — Architecture & Training Loop

Speak

Left side is the architecture block diagram — replay buffer → GRU encoder → actor and six critics → Lagrangian feedback loop. Right side is the per-step pseudocode. Notice that the actor and Lagrangian are updated every 2 critic steps, inheriting TD3's delayed-update trick.

## Slide 17 — Implementation Stack & Methodology

Speak

Python 3.10 + PyTorch 2.x for the agents; FastAPI + WebSocket + React for the live dashboard; Matplotlib + PDFPages for automated reports. Methodology was iterative — DDPG first, then TD3, then the Proposed Algorithm. Each stage's inferences directly fed into the next.

## Slide 18 — Hyper-parameters

Speak

All hyper-parameters live in config.py — single source of truth. Mention HIDDEN_DIM = 512, BATCH_SIZE = 256, LR = 3×10⁻⁴ (standard TD3), POLICY_DELAY = 2. For the Proposed Algorithm, highlight SEQ_LEN = 8, BELIEF_DIM = 16, λ-inits 3.0 / 1.0 / 0.01.

Reviewer might ask

Why λ_1 = 3.0 init? Answer: tuning showed that an initial bias toward throughput prevents a 'zero-power collapse' early on, where the agent collapses into transmitting nothing because it is over-penalised for any violation.

## Slide 19 — Results Setup

Speak

All experiments use imperfect CSI. Three experiments — Experiment A is m=1 (Rayleigh), Experiment B is m=3 (moderate), both TD3 vs DDPG over 3 500 episodes. Experiment C is the three-way comparison at m=3 over 1 500 episodes.

## Slides 20–22 — Experiment A (m = 1)

Speak

Severe Rayleigh fading. TD3 wins on average reward (310.75 vs 272.07), SU throughput (2.37 vs 2.07), outage (0.249 vs 0.321) and BER (0.059 vs 0.077). DDPG only wins on PU throughput because it transmits more conservatively. Even TD3 cannot push outage below 25 % — a structural consequence of deep Rayleigh fades, not the agent.

## Slides 23–25 — Experiment B (m = 3)

Speak

Moderate fading. TD3 roughly halves DDPG's outage (0.11 vs 0.25) and improves SU throughput by ~22 %. Convergence is markedly smoother than under m = 1. This is the regime where TD3 really shines.

## Slides 26–28 — Experiment C (3-way, m = 3)

Speak

Under identical settings, the Proposed Algorithm achieves the highest average reward (247.6) and the highest PU throughput (0.529), while tying DDPG on SU throughput, outage and BER — without any manual weight tuning. The cost is training time, which is ~8× longer than TD3 because of the GRU encoder, six critics and Lagrangian updates.

Killer point

The Proposed Algorithm achieves the highest PU throughput at equal outage — safer interference behaviour at equal SU performance. This is the headline contribution.

## Slide 29 — Discussion

Speak

Three headline findings — DDPG → TD3 → Proposed gives monotonic improvement; TD3 alone beats DDPG by 17–22 % at m = 3; the Proposed Algorithm dominates on the cost-aware metric (PU protection at equal SU rate). Fading severity matters — m = 1 is hard for everyone because deep fades dominate; m = 3 is where the agent's intelligence pays off.

## Slide 30 — Conclusion & Future Work

Speak

Phase 2 delivered all three algorithms, benchmarked them under imperfect CSI at m = 1 and m = 3, and showed the Proposed Algorithm's advantages. Future work spans multi-SU MARL, SAC baseline, prioritised replay, transfer learning, path-loss + mobility, OFDM extension, spectrum-sensing coupling and Docker + Optuna engineering.

## Slide 31 — References

Speak

Briefly mention the key three — Zhong et al. 2022 (IRS + CRN), Teja et al. 2026 (Imperfect CSI D2D) and the foundational Fujimoto (TD3) and Lillicrap (DDPG) papers.

## Slide 32 — Thank You

Speak

Thank you. We welcome your questions and suggestions.

# 8.  Anticipated Reviewer Q&A — Model Answers

Reviewers tend to ask one of four kinds of question — conceptual (what is X?), comparative (why X and not Y?), implementation (how did you implement X?), and results (why are the numbers what they are?). Below is a bank of likely questions with concise, defensible answers.

## 8.1  Conceptual

Q.  What exactly is the difference between Underlay and Overlay CRN?

A.  In Underlay, the SU coexists with the PU on the same band by keeping interference below a threshold (continuous power control, no cooperation needed). In Overlay, the SU has prior knowledge of the PU codebook and actively relays / pre-codes the PU signal — strong cooperation assumption that gives a theoretical upper bound.

Q.  Why is the SINR formula different at the PU and SU?

A.  Because each receiver sees different signals. SINR at the PR is (P_p · |h_pp|²) / (P_s · |h_sp|² + σ²) — the PU signal is its own and the SU signal is interference. SINR at the SR is (P_s · |h_ss|²) / (P_p · |h_ps|² + σ²) — symmetric, with the PU as the interferer.

Q.  Why imperfect CSI?

A.  Real receivers estimate channels from pilot symbols, so the agent cannot observe the true channel gains. Imperfect CSI is the realistic deployment scenario, and the project deliberately trains under it so that the policy transfers to real systems.

Q.  What is the Markov property and why does it matter?

A.  Markov means the future depends only on the current state, not the history. It is the assumption that lets RL theory apply. In our setup channels are i.i.d. block-fading and P_s(t−1) is in the state, so the Markov property holds.

## 8.2  Algorithmic

Q.  Why TD3 and not SAC or PPO?

A.  TD3 is the strongest deterministic off-policy continuous-control algorithm and the natural successor to DDPG. SAC is similar performance with entropy regularisation — a good future-work comparator. PPO is on-policy, so it would need many more samples to match TD3 on this problem.

Q.  What does 'overestimation bias' mean and how does TD3 fix it?

A.  A single critic trained with bootstrapped targets and noise tends to systematically over-estimate Q-values because the max operator picks up noise positively. TD3 trains two critics independently and uses min(Q1, Q2) in the target — tighter, less optimistic estimates that produce cleaner actor gradients.

Q.  Why a GRU and not just a wider feed-forward network?

A.  A feed-forward network on the 7-D state has no concept of time. A GRU sees the last 8 observations as a sequence and learns short-term temporal patterns (recent fades, action consequences). The 16-D belief vector is concatenated with the current state so the actor / critic gets both static and temporal information.

Q.  How does the Lagrangian learning rate stay stable?

A.  η_λ = 5×10⁻⁴ is an order of magnitude slower than the actor / critic learning rate. We also clamp λ_k to [0.1, 20.0] so no objective is fully ignored and no runaway happens. Softplus parameterisation guarantees positivity.

Q.  Why is directional noise μ_bias = −0.05 and not larger?

A.  We tuned this. μ_bias = −0.15 was too aggressive — it drove early actions to zero and caused a 'don't transmit' collapse. μ_bias = −0.05 nudges toward safer actions without saturating; the multiplier (1 + v_t) strengthens the bias when violations actually occur.

## 8.3  Implementation

Q.  How are channel gains generated each step?

A.  Each gain is drawn independently from a Gamma(shape = m, scale = Ω/m) distribution using NumPy. This is the standard Nakagami-m model. Block fading means each gain stays constant for one channel use and is re-drawn next step.

Q.  How is the replay buffer implemented?

A.  Pre-allocated tensors on the GPU, capacity 200 000 transitions. Pre-allocation avoids CPU-to-GPU transfer overhead. Sampling is uniform random, batch size 256. The Proposed Algorithm uses a sequence replay buffer that also stores the last 8 observations per transition for the GRU encoder.

Q.  Is BER simulated bit-by-bit or computed analytically?

A.  Computed analytically using the instantaneous BPSK formula BER = 0.5 · erfc(√SINR). Bit-level simulation would be more accurate but enormously slower; the analytical formula is the standard textbook approximation.

Q.  What does the WebSocket dashboard show?

A.  Animated SVG network topology with live channel gains, BER vs SINR scatter plot with theoretical Nakagami-m curves, SU throughput time series, outage probability and a stats panel. It updates every 50 environment steps via a FastAPI WebSocket.

## 8.4  Results

Q.  Why is DDPG's PU throughput higher than TD3's in Experiment A?

A.  Because DDPG transmits more conservatively (lower average P_s). Lower SU power means less SU-to-PU interference, so SINR_p stays higher and PU throughput is preserved — but at the cost of SU throughput. It is not that DDPG is 'better at PU protection' in any sophisticated sense; it is just being timid.

Q.  Why can't TD3 push outage below 25 % at m = 1?

A.  Structural property of Rayleigh fading. With m = 1, |h_ss|² can collapse to near zero unpredictably; no choice of P_s within [0, P_max] can rescue throughput in those deep fades. The agent is bandwidth-limited by the channel, not by its own intelligence. m = 3 has lighter tails so the agent can actually exploit good states.

Q.  In Experiment C the Proposed Algorithm and DDPG tie on SU throughput and outage — does that mean the new mechanisms are wasted?

A.  No. The headline metric is PU throughput at equal outage — Proposed wins (0.529 vs 0.516) without any manual weight tuning. Equivalently: the Proposed Algorithm achieves the same SU performance with safer interference behaviour, automatically. Translate to a deployment scenario where the operator changes γ_th — the Proposed Algorithm self-adapts; DDPG / TD3 would need re-tuning.

Q.  Why is the Proposed Algorithm 8× slower per episode?

A.  Six critics × two networks each = 12 critic forward / backward passes per step (vs 2 in TD3). Plus the GRU encoder forward pass. Plus the Lagrangian dual update. Inference cost is unchanged — only training scales up.

# 9.  Quick-Reference Cheat Sheet

The numbers and one-liners to have at the tip of your tongue.

### 9.1  Headline Numbers (memorise)

| Quantity | Value |
| --- | --- |
| Project — name in one line | Power Allocation in CRN using Deep RL |
| Topology | 4-node underlay (PT, PR, ST, SR) |
| Channel model | Nakagami-m block fading, Ω = 1.0 |
| Fading severities tested | m = 1 (Rayleigh) and m = 3 (moderate) |
| CSI assumption | Imperfect (noisy estimates) |
| P_p / P_max / γ_th | 1.0 W / 3.0 W / 1.0 (≈ 0 dB) |
| Algorithms | DDPG, TD3, Proposed Algorithm |
| Number of critics (Proposed) | 6 (twin × 3 objectives) |
| Belief dim / SEQ_LEN (Proposed) | 16 / 8 |
| Lagrangian inits (λ₁/λ₂/λ₃) | 3.0 / 1.0 / 0.01 |
| Hidden dim / batch size | 512 / 256 |
| Learning rate (actor & critic) | 3 × 10⁻⁴ |
| Replay buffer size | 200 000 |
| Steps per episode / training episodes | 200 / 1 500–3 500 |
| Exp A — m=1 TD3 SU rate | 2.365 bits/s/Hz vs DDPG 2.065 |
| Exp A — m=1 TD3 outage | 0.249 vs DDPG 0.321 |
| Exp B — m=3 TD3 SU rate | 2.067 bits/s/Hz vs DDPG 1.693 |
| Exp B — m=3 TD3 outage | 0.113 vs DDPG 0.251 |
| Exp C — m=3 3-way Avg Reward | Proposed 247.56 > DDPG 245.46 > TD3 241.33 |
| Exp C — Proposed PU rate (advantage) | 0.529 vs DDPG 0.516 |

### 9.2  Elevator Pitch (60 seconds)

Cognitive Radio Networks let unlicensed Secondary Users opportunistically share licensed spectrum with Primary Users. The hard sub-problem is dynamic power allocation under Nakagami-m fading with imperfect CSI — the SU must choose its transmit power every channel use to maximise its own throughput while keeping the PU's SINR above a protection threshold. We implemented three Deep Reinforcement Learning algorithms — DDPG as a Phase-1 baseline, TD3 as a stronger off-policy continuous-control method, and a Proposed Algorithm that extends TD3 with a GRU belief encoder, decomposed multi-objective critics, adaptive Lagrangian weights and directional exploration noise. Across Nakagami m = 1 and m = 3 under imperfect CSI, TD3 substantially outperforms DDPG (+20 % SU throughput, −12 percentage-point outage at m = 3), and the Proposed Algorithm further matches the best baseline on SU performance while improving PU protection — without any manual reward-weight tuning.

### 9.3  Do's and Don'ts on the Day

- DO expand every acronym the first time you say it (CRN, RL, DDPG, TD3, GRU, etc.).

- DO speak to the metric, not the colour — say 'TD3 throughput' not 'the orange line'.

- DO point to the graph when quoting a number — reviewers like to see the source.

- DO own the trade-offs — 8× longer training is a real cost; admit it openly.

- DON'T claim the Proposed Algorithm is 'better at everything' — be precise: best avg reward, best PU rate at equal outage.

- DON'T promise multi-SU / OFDM as 'done' — they are future-work items.

- DON'T mention internal names like 'CAMO-TD3' — the deck calls it the 'Proposed Algorithm'.

### 9.4  One-Sentence Justifications

| Choice | Justification |
| --- | --- |
| Underlay paradigm | Most general, continuously active, central sub-problem is power control. |
| DDPG → TD3 → Proposed | Each algorithm addresses a specific failure mode of the previous one. |
| Continuous action | DQN's discrete bins are too coarse for fine-grained P_s tuning. |
| Nakagami-m fading | Single-parameter family that includes Rayleigh and AWGN as special cases. |
| Imperfect CSI | Realistic deployment scenario; perfect CSI is an oracle assumption. |
| GRU over LSTM | Fewer parameters, matches LSTM at SEQ_LEN = 8. |
| 6 critics (decomposed) | Cleaner per-objective Q-signal enables explicit trade-offs. |
| Adaptive Lagrangian | Primal-dual formulation of CMDP — removes hand-tuned reward weights. |
| Directional noise | Safer early exploration; bias decays to zero so final policy is undistorted. |
| TF32 on Ampere/Ada | Free ~2× speed-up at no accuracy cost. |